Exact reset-to-auto-calibration workflow for your current firmware:

- Power/reset the ESP32, then open Serial Monitor at 115200 baud.

- On boot, setup runs and loads EEPROM values (or defaults if EEPROM is missing/invalid), then starts startup empty detection.

- During startup empty detection:

- Firmware waits up to 15 seconds for a stable empty scale.

- Empty is considered stable when readings stay within ±1.5 lbs for 3 checks.

- You can send q to skip startup tare.

- If stable is detected, it tares automatically.

- If unstable at timeout, it continues without tare and tells you to use r later.

- After boot messages, send a to enter automatic calibration mode.

- Automatic calibration then does this sequence:

- Verifies HX711 is ready.

- Requires empty scale confirmation (auto-detect, up to 30 seconds, q cancels).

- If this empty confirmation fails/cancels, calibration factor is reset to default and saved.

- If empty is confirmed, it clears scale factor and tares.

- Computes load detect threshold as max(noise × 20, minimumThresholdLbs), where noise is a short averaged no-load read.

- Prompts you to place the known reference weight.

- Waits up to 15 seconds for the placed weight to exceed the computed threshold.

- Takes a stable averaged reading, computes calibration factor as measuredUnits / knownWeightLbs.

- Applies the new factor, prints a verification reading, and saves to EEPROM.

Current known-weight value in your config is 36.8 lbs, so place that exact reference mass when prompted.

Key implementation references:

- Startup and command flow: PropaneScale.ino:1306

- Auto-calibration routine: PropaneScale.ino:655

- Calibration constants (timeouts, known weight, thresholds): config.h:1

If you want, I can also give you a short operator checklist version (just the physical actions and expected serial prompts).