Auto-Cal Operator Checklist (From Reset)

- Ensure the scale platform is empty.

- Power-cycle or reset the ESP32.

- Open Serial Monitor at 115200 baud.

- Watch startup:

- If it auto-tares, continue.

- If it says it continued without startup tare, remove all weight and send r once to re-zero.

- When command menu appears, send a.

- Keep scale empty while auto empty-confirm runs (up to ~30s).

- When prompted, place the known calibration weight (36.8 lbs) on the scale.

- Keep the weight still until it reports detection and computes the factor.

- Confirm messages indicate:

- Calibration factor computed.

- Verification reading printed.

- Calibration saved to EEPROM.

- Remove weight.

- Optional validation:

- Send l and place your tank to verify reasonable propane reading.

- Send e to confirm saved calibration value in EEPROM.

Abort/Recovery Quick Rules

- If empty confirmation times out/cancels, calibration resets to default and saves it; send a again after stabilizing the scale.

- If weight placement times out, restart by sending a and place weight only when prompted.

- If readings look wrong after calibration, send r on empty scale, then repeat a.